KYRA Person Height Measurement Module

Complete Technical Documentation

Project Overview The KYRA Person Height Measurement Module is a fully marker-free, single-webcam height measurement system built in Python using MediaPipe Pose and OpenCV. It estimates the real-world height (in centimetres) of a standing person using only a standard USB or laptop webcam — no depth sensors, no AR markers, no external hardware.

Key Features:

• Marker-free detection — uses AI body keypoints (no physical markers)

• Two calibration methods — calibrate with a physical object OR directly using a person of known height • On-screen visual feedback — height, position status, and detecting indicators overlaid on the live camera view

• Rolling median smoothing — reduces measurement jitter using a 10-frame rolling median

• Persistent calibration — scale factor is saved to a JSON config file; works across sessions and across different locations (home, office)

• Validation suite — automated accuracy sweep with PASS/FAIL report

• Distance sensitivity testing — benchmarks accuracy across close, medium, and far distances

Module Descriptions:

1. landmarks.py

Purpose: Uses MediaPipe Pose to detect body keypoints and compute precise head-top and feet-ground pixel coordinates.

Key Design Decisions:

• Head Top (Dynamic Scaling): The head top is estimated by shifting the nose point upward by 50% of the nose-to-shoulder vertical distance. This is anatomically correct and automatically scales for short and tall people, eliminating the 2–3 cm height-dependent bias that a fixed pixel offset would create

• Feet Ground Point (Absolute Lowest): The feet Y-coordinate is the maximum Y value (lowest point on screen) among 6 landmarks: left/right ankles (27, 28), left/right heels (29, 30), and left/right toes (31, 32). This ensures the tracking line reaches the floor contact level.

• Shoulder Visibility Check: Both shoulders must be detected at ‡ 50% confidence for the dynamic head-shift calculation to work.

Output: {"head": (x, y), "feet": (x, y)} pixel coordinates, or None if landmarks are not visible.

2. height_webcam.py

Purpose: Calculates physical height in centimetres from pixel coordinates using a pre-calibrated pixels-per-cm scale factor.

Height Calculation Formula:

height_cm = (|head_y - feet_y| / pixels_per_cm) + 1.0 The +1.0 cm is a residual correction for minor perspective projection and hair volume effects.

Calibration Storage (config/height_calibration.json):

{ "pixels_per_cm": 8.52,

"calibrated_base_y": 447,

"saved_at": "2026-07-15T14:30:00"

}

• pixels_per_cm — the core scale factor

• calibrated_base_y — the Y-pixel of the floor level during calibration (used for position guidance)

Key Functions:

3. height_stereo.py

Purpose: Provides stereo-camera height estimation using RANSAC floor-plane fitting on 3D point clouds.

Method:

1. Takes an RGB-D point cloud from stereo calibration.

2. Fits a ground plane using RANSAC to identify the floor level.

3. Measures the perpendicular 3D distance from the head point to the fitted floor plane.

4. Subtracts a 1.5 cm hair volume correction.

Key Functions:

4. target_detector.py

Purpose: Wraps landmark-based and ROI-based detection into a common interface for the universal pipeline

5.pipeline.py

Purpose: The main CLI entry point. Orchestrates camera setup, calibration, mode routing, and live display.

Key Functions:

6. validate.py

Purpose: Automated accuracy evaluation suite with structured JSON logging

Output JSON Log Structure:

{

"summary":

{ "mode": "webcam",

"known_height_cm": 146.0,

"mean_measured_height_cm": 146.8,

"std_dev_cm": 0.3,

"absolute_error_cm": 0.8,

"error_percentage": 0.55,

"passed": true,

"n_samples": 20 },

"samples": [...]

}

How It Works

Step 1: Camera Frame

Capture Every frame from the webcam is captured as a BGR NumPy array via cv2.VideoCapture.

Step 2: Landmark Detection (MediaPipe Pose)

The frame is converted from BGR to RGB and processed by MediaPipe Pose (model_complexity=1).

This returns 33 body landmarks with normalised (0–1) x, y coordinates and visibility scores.

Step 3: Head Top Estimation

shoulder_y = mean of left_shoulder.y and right_shoulder.y

shift = 0.50 x |nose.y - shoulder_y| x frame_height

head_y = nose.y x frame_height – shift

This is proportional to the person's on-screen size — automatically correct for any height or distance.

Step 4: Feet Ground Estimation

foot_y = max(

left_ankle.y, right_ankle.y,

left_heel.y, right_heel.y,

left_toe.y, right_toe.y

) x frame_height

This finds the absolute lowest visible point of the foot, reaching the actual floor contact level.

Step 5: Height Calculation

pixel_span = |head_y - foot_y|

height_cm = (pixel_span / pixels_per_cm) + 1.0

Step 6: Rolling Median Smoothing

The height is appended to a rolling deque of the last 10 values. The on-screen display always shows the median of this window to filter out single-frame jitter

Step 7: Position Guidance

The saved calibrated_base_y is compared to the current feet[1]:

Calibration Methods

Method A: Person Self-Calibration (Recommended)

Command: python src/height/pipeline.py --ref-person-cm (enter actual height)

How it works:

1. The camera detects your full body in the frame.

2. It measures your current pixel span (head to feet).

3. It solves for pixels_per_cm such that the formula outputs exactly your known height: pixels_per_cm = pixel_span / (ref_height - 1.0)

4. The result is saved to config/height_calibration.json.

Why this is most accurate: The scale factor is derived from your exact body at your exact standing distance, with zero perspective error.

Method B:Object Calibration

Command: python src/height/pipeline.py --ref-object-cm (object height)

How it works:

1. Camera live feed opens. Press SPACE to freeze the frame.

2. Draw a bounding box from the top to the bottom of the reference object.

3. Press ENTER to save.

4. The calibration computes: pixels_per_cm = box_height_pixels / ref_object_cm

5. Both the scale and the base Y-coordinate (ry + rh) are saved.

Important: Place the reference object on the floor on the exact standing spot. Holding it in your hand will cause a perspective error because your hand is closer to the lens than your feet.

Calibration Portability

The calibration file is stored at config/height_calibration.json.

You can:

• Re-run calibration in a new location (office, home) at any time — it overwrites the old file.

• Use the same calibration file as long as the camera position and angle remain unchanged.

• Calibrate once per measurement session/location for the best accuracy.

On-Screen Indicators

Execution Commands

1.Calibration — Option A: Person Self-Calibration (Most Accurate)

python src/height/pipeline.py --ref-person-cm 146.0 (Replace 146.0 with your actual height in cm.)

Steps:

1. Stand straight on the marked floor spot with your full body visible.

2. The screen shows SPACE: Calibrate to 146.0 cm.

3. Press SPACE to capture. Calibration saves and the window closes automatically

2. Calibration — Option B: Object Calibration python

src/height/pipeline.py --ref-object-cm 27.5 (Replace 27.5 with your reference object's actual height in cm.)

Steps:

1. Place the object vertically on the floor at the exact standing spot.

2. Live camera opens. Press SPACE to freeze the frame.

3. Click the top-left corner of the object and drag diagonally to the bottom-right corner.

4. Press ENTER to confirm and save.

3. Live Person Height Measurement

python src/height/pipeline.py --mode webcam --target person

Steps:

1. Stand on the calibrated floor spot.

2. Check the top overlay — wait for POSITION: OK (green).

3. Stand straight. The bottom of the screen shows your measured height.

4. Press Q or ESC to exit.

4. Live Object Height Measurement

python src/height/pipeline.py --mode webcam --target object

Steps:

1. Place the object you want to measure on the floor spot.

2. The live camera opens. Press SPACE to freeze.

3. Draw a bounding box around the object from top to bottom.

4. Press SPACE again to calculate and display height.

5. Press Q to exit

5. Validation Sweep (Accuracy Test) Technical Documentation

python src/height/validate.py --mode webcam --known-height 146.0 --samples 20

(Replace 146.0 with the test person's actual height.)

Steps:

1. Stand on the calibrated spot. Wait for POSITION: OK.

2. The program automatically collects 20 valid frames.

3. Results printed on screen and saved to src/height/validation_logs/. 8.6 Distance Sensitivity Test python src/height/validate.py --mode distance-test --known-height 146.0

Steps:

1. The program prompts you to stand at close (~1m), medium (~2m), then far (~3m).

2. Press ENTER at each prompt to start collecting samples.

3. A PASS/FAIL table is printed comparing each distance bucket.

7.Manual Scale Override (No Calibration)

python src/height/pipeline.py --mode webcam --target person --manual-scale 10.0

Validation and Testing

Accuracy and Known Limitations

Accuracy Under Ideal Conditions

Factors That Improve Accuracy

• Plain, contrasting background (white or grey wall)

• Camera at chest height, lens level (not tilted up or down)

• Good frontal lighting (avoid bright windows behind person)

• Full body visible — head top to toes in frame

• Person standing straight and still on the calibrated spot

Known Limitations

PHASE 2

## 2. The Core Problem:

When a user stands in front of the mirror wearing loose, baggy, or draped outfits (such as oversized t-shirts, hoodies, wide-leg trousers, or skirts), the silhouette extraction pipeline fails to distinguish between the fabric boundaries and actual anatomical boundaries.

This results in a severe inflation of the visual contour, causing the deep learning model to output inaccurate measurements.

THANK YOU